Here are a few strings of NMOS inverters.

NMOS is harder to simulate than CMOS.

nmos22.ckt	all one power supply

nmos30.ckt

nmos100.ckt

nmp100.ckt	separate power supplies

The number is the number of gates.

Power supply coupling can hinder convergence, so the big circuits

have separate power supplies for each gate.  At 1000 gates it makes

the difference between convergence and non-convergence.  At 100

gates, it slows it down a bit but doesn't change the number of

iterations.  The latency exploitation doesn't work as well with

common power supplies.  The reason for the problem is that the

voltage source in ACS is non-ideal.  A future release will have an

ideal voltage source.

Still... Spice 2g6 doesn't converge on either 100 gate circuit.

It takes 45 iterations compared to 25 in ACS for the same results.

Another point is that these large circuits show why ACS doesn't

automatically print everything when you do an "op" analysis.

Everything Spice gives you is available.  We just don't print it

automatically.

The two .c files nmosgen.c and nmpgen.c are what I used to generate

these circuits.