Here are two variations on a CMOS op-amp, similar to the sample 2

stage one in Allen & Holberg's "CMOS Analog Circuit Design" text.

It is not an optimal design, just one to play with.  The results

are the same as Spice, as expected.

The files:

opamp-ol.ckt	open loop: op, ac, dc

opamp-vf.ckt	voltage follower: op, ac, dc, tran

opamp-nc.ckt	voltage follower: with a convergence problem

We do a few extras, like a plot of Cgs(M7) vs. input voltage.  Try

that in Spice.

Then, there's another one.  An ac analysis at saturation.  Try that

in Spice.

"opamp-nc.ckt" has a known convergence problem that can be solved

by allowing more iterations.  I left it here because ACS continues,

giving reasonable results everywhere else, without crashing.

Spice shows you everything, whether you like it or not, on an "op"

analysis.  You can't see them at all in any other.  But those values

are not constant.  They vary with the signals.  ACS will show you

(if you want) how they vary with the signal.